PLAN: Using Graph Neural Networks to Estimate Atmospheric Parameters in the Scandinavian Thermosphere

Abstract

Introduction

- Introduce the classic issue of the need for better thermospheric parameter prediction for satellites in LEO due to the hockey-stick curve and the need to avoid Kessler syndrome.

- Discuss the current model types in use (empirical and physics-based), how they are deployed, and give a brief literature review on their accuracy (quantify) and why they need to be better – i.e. how their inaccuracy affects orbit propagation.

- Introduce the new age of ML/NN research, and the previous, recent success in many fields, including the upper atmosphere.

- Bring forward the novel approach of GNNs, why they have been chosen, and how they could be useful to the community (brief).

- Introduce the idea of data assimilation and how having a more accurate network of temperature and wind speed predictions could be integrated into larger physics-based models for orbit propagation.

Background

- Discuss Navier-Stokes, hydrostatic balance equations to explain how predicting temperatures and wind speeds at 200-250 km affects LEO satellites (slightly higher altitude).

- Explain how current atmospheric models calculate these parameters and their limitations.

- Explain how GNNs have the potential to be superior, referencing past literature and referring to their formulation (NN architecture, node-edge, etc). Also, explain their superiority over standard ML/NN architectures.

- Describe and explain how an increase in observations/model output in any region, but in particular high-latitude regions, is beneficial in data assimilation and thus provides more accurate orbit propagation.

- Introduce the Fabry-Perot Interferometer, how it works, its accuracy, and how its data is underused, and should be incorporated into atmospheric state prediction algorithms, particularly as there is such a rich network of observations in the Scandinavian region due to UCL.

## Methodology

### Data Processing for FPI, IMF, and Geomagnetic Data Sources

- The study utilises Fabry–Pérot Interferometer (FPI), Interplanetary Magnetic Field (IMF), and geomagnetic index data obtained from publicly available scientific repositories, including instrument-specific data archives and space weather databases.

- The raw datasets are provided in heterogeneous formats (e.g., CSV, CDF, and NetCDF) and are converted into a unified, machine-readable format through standardised parsing, temporal alignment, and normalisation procedures to create a dataset suitable for input into a Graph Neural Network (GNN).

- Data profiling is conducted to identify missing values, outliers, temporal inconsistencies, and measurement anomalies. Decisions such as interpolation, filtering, or data exclusion are made based on physical plausibility, data coverage, and their potential impact on model stability.

- The processed datasets are merged using a common temporal reference, ensuring synchronised observations across all data sources while preserving spatial and physical relationships relevant to upper-atmospheric dynamics.

- Data validity is ensured through consistency checks, comparison with known physical bounds, and cross-referencing overlapping measurements across independent data sources.

### PyTorch GNN Data Object Creation

- The PyTorch Geometric data object is used to represent the graph structure, consisting of node features, edge indices, and optional edge attributes and labels.

- The processed datasets are transformed into PyTorch data objects by mapping physical measurements to node features, defining edges based on spatial, temporal, or physical relationships, and encoding target variables for supervised learning.

- These data objects are ingested into the GNN architecture through PyTorch Geometric data loaders, enabling efficient batching, shuffling, and parallel processing during model training and evaluation.

- Past time-series data for IMF and the Geomagnetic indices are also ingested into the data object, where the number of time steps and the time between them is set by the user prior to training – these are added as columns to the data object

- Principal Component Analysis is performed to reduce the complexity of the dataset for faster computation, whilst obtaining the same accuracy scores.

### GNN Usage on GPU

- GPU acceleration is employed to significantly reduce training time and enable the efficient processing of large graph structures and high-dimensional feature spaces inherent in geophysical datasets.

- Key GNN hyperparameters, including learning rate, number of layers, hidden dimensions, and message-passing depth, are selected based on prior studies, empirical testing, and computational constraints to balance model complexity and generalisation.

- GNN training scripts are executed using PyTorch with CUDA support, allowing for reproducible experimentation and scalable model training.

- Model convergence and peak performance are determined by monitoring validation loss, evaluation metrics, and learning curves to identify saturation or degradation in model improvement.

### Model Validation

- Model validity is assessed through quantitative evaluation metrics and qualitative physical consistency checks to ensure that predictions are both statistically sound and physically meaningful.

- Independent datasets, including WINDII observations, additional FPI measurements, and outputs from the MSIS atmospheric model, are used as external validation sources.

- Model accuracy thresholds are defined based on observational uncertainty and domain-specific requirements, and the extent to which model performance translates to real-world predictive capability is critically evaluated.

Results

- Table of results for different model types, different drivers (e.g. F30 vs F10.7, Dst vs no Dst, etc.)

- Plots for significant storm-periods (e.g. Oct 2003) and for quiet periods to see visually on a time-series graph how it compares to MSIS and other models (could use JB2008, WAM-IPE, TIE-GCM and/or DTM)

- Taylor plots (see Sean Elvidge), for different model types and drivers vs the other models listed above for larger time periods (a year, a solar cycle, solar maximum and solar minimum, etc.)

Discussion

- Analysis of model results vs other models

- Discuss how this would affect orbit propagation

- The need for future iterations, and the ability for this type of network to be usable across the upper atmospheric domain for parameter estimation, highlighting future applications (such as accelerometer/POD/mass-spectrometer density data, etc).

Conclusion

# A Graph Neural Network Approach for High-Fidelity Thermospheric State Estimation and Integration into Operational Space Situational Awareness

The environment of low-Earth Orbit (LEO) is currently experiencing an unprecedented transformation, characterised by the rapid and exponential increase in the population of satellites. Described by the “hockey-stick” curve, growth is primarily driven by the deployment of large-scale commercial satellite mega-constellations, such as SpaceX's Starlink, Eutelsat's OneWeb, and Amazon's Kuiper (Feder, 2022). These companies have projected the deployment of tens to hundreds of thousands of satellites operating predominantly within the critical LEO altitude band of 300–900 km by the end of the decade.

This increased orbital congestion results in a dramatically increased risk of catastrophic collisions. Models developed based on the prediction of collisions indicate that, in specific dense LEO regimes, the rate of new debris generation from random, accidental collisions already exceeds the natural removal rate provided by atmospheric drag (Liou et al., 2006). This imbalance accelerates the potential onset of Kessler Syndrome, where continuous collisions between man-made space objects eventually leave vital orbital regions unusable for decades (Kessler, 2009)

Atmospheric drag is the dominant non-conservative force acting on satellites, with solar radiation pressure being the next most significant, and thruster activity and electromagnetic effects being comparatively minor (Montenbruck, 2000). Drag also serves as the primary natural mechanism for de-orbiting space debris, so it is important in estimating probabilities of collision with other objects in space. Consequently, the accurate prediction of atmospheric drag, which primarily requires precise knowledge of thermospheric density and the position of the target object, is paramount for effective Space Traffic Management (STM) and collision risk mitigation. Addressing the upper atmospheric modelling gap is thus recognised as an immediate, non-regulatory measure available for mitigating systemic collision risk in LEO (Gondelach, 2019).

## Deficiencies in Current Operational Thermospheric Modelling

Operational orbit determination and propagation are largely based on two primary modelling techniques: empirical and physics-based models. Empirical models, such as the Drag Temperature Model (DTM), Jacchia-Bowman (JB2008), and NRLMSIS-2.0, are computationally efficient, relying on statistical fits derived from proxy indices such as F10.7 solar radio flux and geomagnetic Kp/Ap indices (Emmert, 2015). While fast, these models fundamentally struggle to capture the complex, non-linear relationships and localised atmospheric responses, especially during high levels of geomagnetic activity (Bruinsma, 2024).

Physics-based General Circulation Models (GCMs), such as the Thermosphere-Ionosphere-Electrodynamics General Circulation Model (TIE-GCM), and the Whole Atmosphere Model-Ionosphere Plasmasphere Electrodynamics (WAM-IPE), solve fluid equations of the coupled thermosphere-ionosphere system (Richmond, 1992).  GCMs provide physical consistency, but their extensive computational requirements often mean coarse spatial resolutions, such as 5 latitude x 5 longitude for TIE-GCM, which can limit their capacity for high-cadence, real-time operational forecasting. Operationally, these GCMs are typically used in conjunction with a data assimilation (DA) approach, which updates the state using new observations rather than relying solely on model propagation. Data Assimilation is used to optimally merge observational data with numerical model forecasts to achieve an improved analysis state. Assimilating in-situ density observations into physics-based thermospheric models has been shown to reduce root-mean-square errors in neutral mass density estimates by approximately 40–50% relative to non-assimilated model predictions, with improvements validated against independent satellite observations from GRACE and CHAMP (Matsuo et al., 2012).

The inadequacies of current models become critical during geomagnetic storms, which introduce substantial errors into the estimated satellite position and velocity vectors. Comparative studies have shown that during the main and recovery phases of intense storms, all evaluated models (DTM-2020, JB2008, NRLMSIS 2.0, TIEGCM 2.0) consistently exhibit systematic errors, often resulting in an underestimation of neutral density in the main phase, and an overestimation in the recovery phase (Zhang et al. 2025). Even the highest-performing models demonstrate deviations from satellite observations of at least 20% during these periods. These predictive failures lead directly to massive orbit propagation errors and significantly enhanced orbital decay. For example, extreme geomagnetic storms have been shown to increase orbital decay rates by approximately 233% to 300% relative to quiet conditions for missions such as the China Space Station (Zhang et al., 2025), which may not be captured by current models.

The resulting uncertainty in density predictions is not marginal. The 1-sigma uncertainty associated with density, which typically sits between 20% and 30% during quiet periods, exhibits a strong cubic dependency on the magnetic index (Kp), soaring to peaks of up to 60% during periods of high geomagnetic activity (Emmert, 2015). This uncertainty is the dominant source of error in LEO orbit prediction (Vallado et al. 2014), where, in conjunction assessment (CA), this uncertainty directly degrades collision probability estimates. As a result, limitations in current thermospheric models threaten operational safety margins, especially during turbulent conditions.

## The Emergence of Machine Learning in Upper-Atmosphere Modelling

Traditional approaches struggle to represent the complex behaviour of the thermosphere to a high spatial and temporal resolution, leading to growing use of machine learning (ML) and neural networks (NNs) in space weather modelling (Pan et al. 2025). These data-driven models can learn complex input-output mappings without explicit physical parameterisations.

Initial applications of NNs to thermospheric density forecasting have already demonstrated quantitative improvements over operational models. For instance, the MBiLE (Multilayer Perceptron and bi-directional Long Short-Term Memory for ensemble learning) integrated learning model achieved improvements in predictive accuracy compared to the current standard, NRLMSIS-2.0, with an R2 value of 0.912, compared to NRLMSIS-2.0 having a value of 0.89 (Pan et al., 2025). These improvements were consistently validated across various satellite datasets and LEO altitudes, including Swarm C, CHAMP and GOCE data.

Furthermore, the operational acceptance of ML requires confidence in the derived uncertainty volumes. While ML has been criticised as a "black box," recent advances in Explainable AI (XAI), such as SHapley Additive exPlanations (SHAP), have been successfully applied to these models, confirming that they capture well-understood physical drivers (e.g., solar irradiance) while also revealing subtle storm-time thresholds that simple empirical proxies miss (Pan et al. 2025). The integration of uncertainty quantification and explainability establishes the required foundation for adopting ML in operational applications.

## Graph Neural Networks as a Novel Approach

While early ML successes have primarily focused on pointwise or global parameter prediction, they often struggle to encode the inbuilt spatiotemporal relationship of data in the atmosphere (Richsetin et al. 2019). The thermosphere is fundamentally a coupled fluid system where energy and momentum are transferred across magnetic and pressure surfaces, requiring models that capture this dynamic structural relationship.

Graph Neural Networks (GNNs) represent a next-generation methodology, specifically designed to process and analyse data structures defined by relational information, called the ‘graph’. GNNs are suited to model the atmospheric environment by representing the system as a graph, where standard GNNs have two main features:

- Nodes - represent physical locations or observations (e.g., satellite sensor points, ground-based radar sites, or computational grid centres).

- Edges – this is the physical coupling mechanisms, where spatial and temporal distances between observations can be encoded, as well as physical relationships in more advanced models.

By modelling the thermosphere as a graph, GNNs move beyond statistical correlation, allowing the model to explicitly represent atmospheric cause-and-effect relationships, where information flows from one node to the next via learned message passing via edge connections (Sanchez-Gonzalez et al., 2020). This enables the development of models that are physically informed while retaining the speed and predictive power of data-driven approaches.

GNNs have recently demonstrated strong skill in tropospheric weather prediction. GraphCast, a GNN-based global weather model developed by Google DeepMind, has achieved state-of-the-art medium-range forecasts at substantially reduced computational cost, highlighting the ability of GNNs to capture nonlinear dynamics and long-range spatial dependencies (Lam et al., 2023). This framework is extended to GenCast, which produces probabilistic, ensemble forecasts, providing uncertainty representation compatible with data assimilation (Price et al., 2023).

To date, GNN-based forecasting approaches have not been applied to the thermosphere, where strong solar–geomagnetic forcing, sparse observations, and highly variable density directly affect satellite drag. Given their demonstrated efficacy in lower-atmosphere modelling, GNNs represent a promising approach for thermospheric state prediction and satellite drag forecasting, either as a standalone predictive model or when integrated with physics-based models through data assimilation.

## References

Feder, T. (2022) ‘Ballooning satellite populations in low Earth orbit portend changes for science and society’, Physics Today, 1 April. doi: 10.1063/PT.3.4979.

J.-C. Liou, N. L. Johnson Risks in Space from Orbiting Debris.Science311,340-341(2006).DOI:10.1126/science.1121337

Kessler, D.J. and Cour-Palais, B.G. (1978) ‘Collision frequency of artificial satellites: The creation of a debris belt’, Journal of Geophysical Research, 83(A6), pp. 2637–2646. doi: 10.1029/JA083iA06p02637.

Montenbruck, O. and Gill, E. (2000) Satellite Orbits: Models, Methods, and Applications. Berlin: Springer.

Gondelach, David & Linares, Richard. (2019). Atmospheric Density Uncertainty Quantification for Satellite Conjunction Assessment. 10.48550/arXiv.1912.01069.

Emmert, J.T. (2015) ‘Thermospheric mass density: A review’, Advances in Space Research, 56(5), pp. 773–824. doi: 10.1016/j.asr.2015.05.038.

Bruinsma, S. & Laurens, S., 2024. Thermosphere model assessment for geomagnetic storms from 2001 to 2023. Journal of Space Weather and Space Climate, 14, 28. https://doi.org/10.1051/swsc/2024027

Richmond, A.D., Ridley, E.C. and Roble, R.G. (1992) ‘A thermosphere/ionosphere general circulation model with coupled electrodynamics’, Geophysical Research Letters, 19(6), pp. 601–604. doi: 10.1029/92GL00401.

Matsuo, T., M. Fedrizzi, T. J. Fuller-Rowell, and M. V. Codrescu (2012), Data assimilation of thermospheric mass density, Space Weather, 10, S05002, doi:10.1029/2012SW000773.

Zhang, K., Huang, Y., Wei, F., Zuo, P., Yang, H., Ji, J. and Chen, Z. (2025) ‘Thermospheric density variations during extreme geomagnetic storms and their potential impact on the orbit of the China space station’, Frontiers in Astronomy and Space Sciences, 12, 1644152. doi: 10.3389/fspas.2025.1644152.

Vallado, D.A. and Finkleman, D. (2014) ‘A critical assessment of satellite drag and atmospheric density modeling’, Acta Astronautica, 95, pp. 141–165. doi: 10.1016/j.actaastro.2013.10.005.

Pan, Q., Xiong, C., Gao, S., Chen, Z., Smirnov, A., Xu, C., & Huang, Y. (2025). Interpretable machine learning for thermospheric mass density modeling using GRACE/GRACE-FO satellite data. Space Weather, 23, e2024SW004259. https://doi.org/10.1029/2024SW004259

Reichstein, M., Camps-Valls, G., Stevens, B. et al. Deep learning and process understanding for data-driven Earth system science. Nature 566, 195–204 (2019). https://doi.org/10.1038/s41586-019-0912-1

Sanchez-Gonzalez, A., Godwin, J., Pfaff, T., Ying, R., Leskovec, J. and Battaglia, P.W. (2020) Learning to simulate complex physics with graph networks. Proceedings of the 37th International Conference on Machine Learning (ICML 2020). Available at: https://doi.org/10.48550/arXiv.2002.09405

R. Lam et a. Learning skillful medium-range global weather forecasting.Science382,1416-1421(2023).DOI:10.1126/science.adi2336

Price, I., Sanchez-Gonzalez, A., Alet, F., Andersson, T.R., El-Kadi, A., Masters, D., Ewalds, T., Stott, J., Mohamed, S., Battaglia, P., Lam, R. and Willson, M. (2024) GenCast: Diffusion-based ensemble forecasting for medium-range weather. arXiv:2312.15796. Available at: https://doi.org/10.48550/arXiv.2312.15796